# Physics-Informed Machine Learning Approach for NOx Prediction in Indian Subway Tunnel Environments Using Real-Time Air Quality Data

Sudhanshu Ranjan Singh¹

¹ Department of Computer Science / AI & ML

Correspondence: ranjansinghsudhanshu13@gmail.com

Abstract: Air pollution in subway tunnel environments is a growing concern in densely populated cities like Delhi due to pollutant accumulation under limited ventilation. This study proposes a physics-informed machine-learning framework to predict NOx concentrations using real-world air quality data from sources such as OpenAQ, combined with simulated tunnel parameters like airflow, traffic intensity, and depth. Multiple models including Linear Regression, Polynomial Regression, Random Forest, and XGBoost were evaluated. The Random Forest model achieved the best performance with an R² score of 0.988 and RMSE of 7.80. Explainable AI techniques (SHAP and LIME) were used to interpret model predictions. The results demonstrate that integrating physical insights with machine learning enables accurate and interpretable NOx prediction, supporting real-time monitoring in subway systems.

Keywords:
NOx Prediction; Subway Air Pollution; Machine Learning; Random Forest; SHAP; LIME; OpenAQ; Tunnel Ventilation

- Introduction

Air pollution in cities has become a significant problem for the environment and for human health(Liu et al., 2019). As more people move to cities and industries grow, air quality is deteriorating(Kappelt et al., 2022). This is especially true in major urban centers like Delhi, where numerous vehicles are contributing to air pollution(Kumar et al., 2022). While much is known about the pollution on the streets, we do not know as extensively about the air quality in underground transportation systems like metro trains. These systems are important for facilitating urban mobility, but they can also trap pollutants in a confined spaces, exacerbating air pollution over time(Lin et al., 2023).

Subway tunnels are unique due to their limited fresh air(Lin et al., 2023) circulation. This is different from being outside in the city where the air can move around and dispersed pollutants(Martins et al., 2017). In tunnels, the polluted air becomes trapped and cannot escape, making it worse for people inhaling it(Wu et al., 2022). Numerous factors contribute to air pollution in tunnels, such as train operations, braking systems, and various operational equipment(Ličbinský et al., 2013). Even pedestrian movement within the tunnels can degrade air quality(Samad et al., 2022). Furthermore, ambient air pollution can infiltrate the tunnels through vents and entrances, further exacerbating the overall pollution levels (Lin et al., 2023) Figure 1

1

Figure 1: Pollutant accumulation and airflow behaviour in subway tunnel environments

Maintaining good respiratory health is not always straightforward, especially in crowded places like subways. Various airborne pollutants pose health risks, with nitrogen oxides, specifically nitric oxide and nitrogen dioxide, being among the most concerning(Wang et al., 2024). These gases are detrimental to human health, contributing to issues such as respiratory distress, impaired lung function, and increased susceptibility to illness(Bairagee et al., 2023). Prolonged exposure can also lead to chronic health conditions such as asthma and bronchitis(Bhat et al., 2024). Beyond direct health impacts, these gases also contribute to atmospheric degradation by facilitating the formation of secondary pollutants like ozone and particulate matter(Bhat et al., 2024). Within confined environments such as subway tunnels, the risk is exacerbated due to prolonged inhalation of these harmful substances, posing significant dangers to daily commuters and tunnel personnel.

Given these risks, accurate prediction and monitoring of NOx concentrations in subway systems are critical for ensuring public safety and improving environmental management strategies. Traditional approaches to air quality prediction can be broadly categorized into two types: physics-based models and data-driven machine learning models(Abboud, 2023). Physics-based models rely on fundamental principles such as fluid dynamics, mass balance equations, and chemical kinetics to simulate pollutant dispersion(Nedungadi et al., 2025). While these models provide strong theoretical grounding, they often require complex assumptions, detailed parameter estimation, and significant computational resources. Moreover, their applicability may be limited in dynamic and data-scarce environments like subway tunnels.

Machine learning models provide a different approach to predicting air quality by looking at patterns in past data. Techniques such as Linear Regression, Decision Trees, Random Forest, and Gradient Boosting have demonstrated efficacy in air quality prediction(Kulikova et al., 2023). These models can find relationships between variables that aren't always straightforward, without needing to know the exact physical processes behind them. However, purely data-driven models present their own challenges. Interpreting the rationale behind their predictions can be difficult, and their performance may decline in novel situations with differing physical conditions(Tefera et al., 2025). This is attributed to their limited incorporation of existing domain knowledge.

To overcome these limitations, researchers have explored novel approaches that integrate the strengths of both physical modeling and machine learning(Zideh et al., 2023). These hybrid methods combine the strengths of both techniques by adding domain-specific knowledge to machine learning frameworks(Beucler et al., 2023). By incorporating physics-based features such as airflow dynamics, tunnel geometry, traffic intensity, and meteorological conditions, these models can achieve greater accuracy while maintaining interpretability and reliability. This means they can take into account things like airflow dynamics, tunnel geometry, traffic intensity, and environmental conditions to make better predictions.

In this study, a physics-informed machine learning framework is proposed to predict NOx concentrations in subway tunnel environments. The approach leverages real-world air quality data obtained through the OpenAQ platform(Tian et al., 2024), which provides access to pollutant measurements from monitoring stations such as Shadipur in Delhi(Kamigauti et al., 2024). The dataset includes key pollutant indicators such as carbon monoxide (CO), nitric oxide (NO), nitrogen dioxide (NO₂), ozone (O₃), and particulate matter (PM₁₀), along with meteorological variables like temperature, humidity, and wind speed(Sankar & Krishnamoorthy, 2025).

To make the model better at predictions, we are adding some new features that are based on physical principles(Mulomba & Choi, 2024). These include factors such as how air moves, which influences pollutant dispersion(Hettige et al., 2024), traffic intensity, which indicates pollutant emission rates(Ibrahim & Lyons, 2024), and tunnel depth, which affects how well they can dissipate pollutants(Dolbeault et al., 2024). By integrating these features with real data, the model can understand both how the environment changes and the underlying rules that govern how pollutants behave. Consequently, the model can provide more accurate predictions(Ibrahim & Lyons, 2024).

To better understand what affects the levels of nitrogen oxides, or NOx, in the air, researchers evaluated several machine learning models, including Linear Regression, Polynomial Regression, Random Forest, and XGBoost, to determine which one performed best for predicting NOx levels(Chauhan et al., 2025). They also used special techniques like SHAP and LIME(Wood, 2023) to explain the models' predictions and identify the most important factors. This approach enhanced the transparency of the models' operations and clarified the drivers of NOx levels, providing valuable insights into the problem. By employing these methods, the study aimed to improve transparency and pinpoint the key factors influencing NOx levels, which is crucial for enhancing air quality.

The primary objective of this research is to develop a robust, accurate, and interpretable framework for predicting NOx concentrations in subway environments. Such a system can support real-time monitoring, assist in ventilation system optimization, and contribute to data-driven decision-making for urban air quality(Qin et al., 2026) management. Ultimately, this work aims to bridge the gap between theoretical modeling and practical implementation, offering a scalable solution to address the growing challenge of underground air pollution in modern cities(Noulas et al., 2025).

2. Literature Review

Extensive research has been conducted on subway air quality, tunnel pollution, and machine learning-based environmental modeling. Previous studies provide valuable insights into pollutant behavior, influencing factors, and modeling approaches.

(Zhang, 2024), in Environment International, reviewed indoor subway air quality across multiple cities. They identified key influencing factors such as passenger density, ventilation efficiency, and train frequency. Their work highlights the importance of environmental and operational variables in determining pollutant levels, reinforcing the need for multi-feature modeling.

Additionally, (Banasiewicz, 2023) developed a machine learning model for NOx prediction in underground mining environments. Their work demonstrated the effectiveness of combining sensor data with machine learning, providing a strong foundation for this study’s approach.

(Smith, 2014), published in Elsevier’s Atmospheric Environment, conducted an experimental study on subway platform air quality, focusing on ventilation systems and the piston effect caused by train movement. Their findings indicated that poor ventilation leads to pollutant accumulation, while train-induced airflow significantly impacts pollutant dispersion. This study strongly supports the inclusion of airflow and traffic-related features in predictive models Figure 2.

Figure 2: Influence of ventilation, airflow (piston effect), and traffic on pollutant dispersion in tunnel environments

(Li, 2019), published by MDPI, performed a global analysis of subway pollution and reported that over 70% of subway systems exceed safe pollution limits. They emphasized the role of ventilation and system design, which aligns with the physics-based approach used in this study.

(Kumar, 2020), in Elsevier, analyzed pollution in roadway tunnels and found that traffic emissions are the primary source of NOx, while ventilation controls pollutant concentration. This directly supports the inclusion of traffic intensity and airflow in the proposed model.

Additionally, (Banasiewicz, 2023) developed a machine learning model for NOx prediction in underground mining environments. Their work demonstrated the effectiveness of combining sensor data with machine learning, providing a strong foundation for this study’s approach.

Other studies, including Jung et al. (2017) and Dong et al. (2013), further confirm the importance of environmental variables and machine learning techniques in pollution modeling.

3. Methodology

3.1 Data Collection

Air quality data were obtained from the OpenAQ API(Varshney et al., 2025) for monitoring locations such as Shadipur in Delhi. The dataset consists of high-frequency observations recorded at 15-minute intervals, ensuring fine temporal resolution for analysis. It includes pollutant concentrations such as carbon monoxide (CO), nitric oxide (NO), nitrogen dioxide (NO₂), ozone (O₃), and particulate matter (PM₁₀), along with meteorological variables like temperature, humidity, and wind speed(Pradeep et al., 2024). Figure 3

Figure 3: Data collection pipeline from OpenAQ API to machine learning dataset

This real-world dataset provides the foundation for modeling NOx concentration and simulating subway-like environmental conditions.

3.2 Physics-Informed Feature Engineering and Mathematical Modeling

To improve prediction accuracy and incorporate domain knowledge, we adopted a physics-informed approach(Xu et al., 2025). Instead of relying purely on data-driven learning, fundamental environmental laws and tunnel behavior guided feature engineering(Wang et al., 2024).

3.2.1 Fundamental Formulation (Mass Balance Principle)

The modeling process begins with the mass balance equation(Liu et al., 2023) Figure 4:

$Accumulation=Emission-Removal$(Gao et al., 2025)

Under steady-state tunnel conditions, pollutant concentration can approximate accumulation, leading to:

$C=\frac{E}{V}$(Rafiei, 2024)

where $C$represents NOx concentration, $E$is emission rate, and $V$is the ventilation rate.

Figure 4: Mass balance representation of emission, accumulation, and ventilation

3.2.2 NOx Aggregation from Measured Data

We computed the total NOx concentration using(Rey-Pommier et al., 2022):

$NO_{x}=NO+NO_{2}$(McDowall, 2007)

This provides the primary target variable for prediction.

3.2.3 Emission Modeling (Traffic Influence)

Emission rate traffic intensity directly influences emission rate(Balzotti et al., 2022):

$E∝T⋅EF$(Balzotti et al., 2022)

where $T$represents traffic intensity and $EF$is the emission factor(Chang et al., 2023).
In this study, we approximated traffic intensity as a normalized variable, assuming higher traffic leads to higher emissions.

3.2.4 Ventilation Modeling (Airflow and Wind Effects)

Ventilation plays a critical role in pollutant dispersion(Liu et al., 2025) and is influenced by airflow and wind speed:

$V∝A+W$(Liu et al., 2025)

where $A$is airflow and $W$is wind speed(Noulas et al., 2025).
We derived airflow from wind speed data to represent ventilation efficiency in tunnel conditions.

Figure 5: Influence of airflow and wind speed on ventilation and pollutant dispersion

3.2.5 Tunnel Confinement Effect

Tunnel depth contributes to pollutant accumulation due to restricted dispersion. This effect was modeled as:

$C∝e^{kd}$where $d$is tunnel depth and $k$is a constant representing confinement intensity.(Chang et al., 2025)

3.2.6 Combined Physics-Based NOx Model

By integrating emission, ventilation, and confinement effects, the final core formulation is expressed as:

$C_{NO_{x}}=C_{surface}⋅\frac{(1+\alpha T^{p})e^{\beta d}}{1+\gamma A+\delta W}$(Soulhac et al., 2022)

3.2.7 Environmental Adjustment Factors

To account for atmospheric and chemical influences, temperature and humidity were incorporated:

$C_{NO_{x}}=C_{surface}⋅\frac{(1+\alpha T^{p})e^{\beta d}}{1+\gamma A+\delta W}⋅(1+ηTemp)(1+\mu RH)$(Miech et al., 2023)

3.2.8 Practical Implementation in ML Framework

Based on the above formulations, the following features were engineered Figure 6:

- NOx = NO + NO₂

- Traffic intensity (emission proxy)

- Airflow (ventilation proxy)

- Tunnel depth (confinement effect)

- Temperature and humidity (environmental modifiers)

This approach ensures that the machine learning model learns not only from data but also from real-world physical relationships governing pollutant behavior(Bush et al., 2022)

Figure 6: Influence of airflow and wind speed on ventilation and pollutant dispersion

3.3 Data Preprocessing

The collected data underwent several preprocessing steps to ensure quality and consistency:

- Merged 11 CSV datasets into a unified dataset

- Handled missing values using forward filling techniques

- Converted datetime fields into a standardized format for temporal analysis

- Removed inconsistencies, outliers, and duplicate entries

These steps ensured that the dataset was clean, structured, and suitable for machine learning model training Figure 7.

Figure 7: Data preprocessing workflow for preparing dataset

3.4 Model Development

Four machine learning models were developed and evaluated:

- Linear Regression

- Polynomial Regression

- Random Forest

- XGBoost

The dataset was split into training (80%) and testing (20%) sets. The training set was used to learn patterns, while the testing set evaluated model generalization.

Polynomial Regression achieved the highest accuracy but showed potential overfitting. Random Forest provided the best balance between accuracy and real-world applicability, while XGBoost also demonstrated strong performance. Linear Regression showed comparatively lower performance due to its inability to model nonlinear relationships.(Burnwal & Jaiswal, 2023)

3.5 Project Workflow

The overall workflow of the project consists of the following stages Figure 8:

- Data Processing

  - Real-world air quality data collected from OpenAQ

  - Parameters: NO, NO₂, CO, O₃, PM₁₀, Temperature, Humidity, Wind Speed

- Physics-Based Feature Engineering

  - NOx = NO + NO₂

  - Traffic (simulated emission)

  - Airflow (ventilation proxy)

  - Depth (tunnel confinement)

- Target Variable Construction

  - NOx$_{tunnel}$ derived using emission, dispersion, and accumulation principles

- Model Training and Evaluation

  - Multiple ML models trained and compared

  - Performance metrics: R², RMSE

- Explainability and Validation

  - SHAP used for global feature importance

  - LIME used for local prediction interpretation

- Visualization and Analysis

  - Line plots showing tunnel vs surface NOx

  - Actual vs predicted comparisons

  - Residual analysis for model stability

  - Correlation heatmaps to identify feature relationships

Figure 8: System Architecture of NOx Prediction Framework

4. Results and Analysis

The study evaluated the performance of different models using R² and RMSE metrics. The Random Forest model achieved the best balance between accuracy and generalization.

Table 4.1 presents the performance comparison of different machine learning models for NOx prediction. Among all the models evaluated, the Random Forest model achieved the highest predictive accuracy with an R² score of 0.91, along with the lowest MAE (2.76) and RMSE (3.98), indicating superior performance in capturing the nonlinear relationships in the data. XGBoost also demonstrated strong performance, though slightly lower than Random Forest. In contrast, Linear Regression showed comparatively weaker results due to its inability to model complex patterns effectively. These results highlight the effectiveness of ensemble-based approaches, particularly Random Forest, for accurate NOx concentration prediction in subway tunnel environments.(Chauhan et al., 2025)

4.1 Model Performance Table

Figure 9 summarizes the evaluation metrics of four machine learning models used for predicting NOx concentrations. The comparison provides a quantitative basis for selecting the most suitable model for this study.

Analysis of Model Performance

Optimal Performance (Random Forest):
The Random Forest model achieved the best overall performance, with the highest coefficient of determination (R² = 0.91) and the lowest error values (MAE = 2.76, RMSE = 3.98). This indicates its strong ability to capture complex nonlinear relationships and generalize effectively in tunnel environments.

Gradient Boosting (XGBoost):
XGBoost also demonstrated strong predictive capability (R² = 0.89), though slightly lower than Random Forest. This marginal difference may be attributed to dataset characteristics, noise sensitivity, or hyperparameter configuration.

Baseline Models:
Linear Regression showed the lowest performance (R² = 0.72), confirming that linear models are insufficient for modeling the nonlinear dynamics of pollutant dispersion. The Decision Tree model (R² = 0.81) performed moderately well but is more prone to overfitting due to higher variance compared to ensemble methods.

Error Sensitivity:
For all models, RMSE values are higher than MAE, indicating the presence of occasional large prediction errors. This suggests the existence of outliers or sudden spikes in NOx concentration (as observed in Fig. 4(a)). Among all models, Random Forest handled these variations most effectively.

Discussion Linkage

These results directly support the finding that while simpler models provide baseline understanding, ensemble methods—particularly Random Forest—offer superior performance. The improvement of approximately 0.19 in R² between Linear Regression and Random Forest highlights the importance of selecting models capable of capturing nonlinear environmental interactions.

Figure 9: Comparative Performance Metrics of Machine Learning Models

- Graphical Analysis

- Time-Series Analysis of NOx Concentration

This time-series plot compares NOx concentration levels measured at the surface and within the tunnel environment over a three-month period from January to March 2026 Figure 10.

Key Observations

Magnitude Disparity:
Tunnel NOx concentrations (orange) consistently exhibit significantly higher values compared to surface levels (blue). While surface concentrations remain relatively stable and lower, tunnel concentrations frequently show sharp peaks exceeding 400 and even 800 units. This behavior is attributed to the accumulation of vehicular emissions in a confined tunnel environment with limited ventilation.

Synchronous Trends:
Both surface and tunnel datasets display synchronized temporal patterns. Increases in surface NOx levels are typically followed by corresponding increases in tunnel concentrations, albeit with a much higher magnitude. This indicates that ambient outdoor air quality and regional traffic patterns influence both environments simultaneously, with amplification occurring inside the tunnel.

Extreme Volatility:
Tunnel NOx data demonstrate high variability, characterized by rapid and short-duration spikes. These fluctuations are indicative of peak traffic periods or the passage of heavy-duty vehicles, which generate transient but intense pollution events within the confined tunnel space.

Data Gaps:
Linear segments observed during mid-January indicate potential missing data or sensor downtime. Such gaps are common in long-term environmental monitoring and highlight the importance of robust data preprocessing techniques.

Figure 10: Temporal Comparison of Surface vs. Tunnel NOx Concentrations

- Actual vs Predicted (Random Forest)

This scatter plot illustrates the performance of the Random Forest regression model in predicting NOx tunnel concentrations. The y=x reference line represents a perfect prediction scenario where the predicted values exactly match the observed data Figure 11.

Analysis of Model Performance:

- High Correlation: The majority of data points are tightly clustered along the diagonal identity line, indicating that the model captures the underlying variance of the NOx emissions with high precision.

- Predictive Accuracy: The model demonstrates strong performance across both low and high concentration ranges, maintaining linearity throughout the dataset.

- Outlier Identification: A few minor deviations are visible (notably near the 200 and 400 actual NOx marks) suggesting specific environmental or operational conditions where the model slightly under-predicted the concentration.

- Overall Fit: The close alignment between the predicted and actual values suggests a high Coefficient of Determination (R^2) validating the Random Forest approach as an effective tool for this application.

Figure 11: Actual vs. Predicted NOx Tunnel Concentrations

- Residual Plot

The residual plot provides a diagnostic evaluation of the model’s error distribution. By plotting the residuals (the difference between actual and predicted values) against the predicted NOx levels, we can assess the reliability of the Random Forest regressor Figure 12.

Key Observations:

- Homoscedasticity: The majority of the residuals are concentrated near the zero-horizontal line across the entire range of predicted values. This indicates that the model maintains a relatively consistent error variance, a sign of robust fit.

- Error Distribution: The dense clustering of points between -25 and +25 units suggests that the model's predictions are highly accurate for the bulk of the dataset.

- Outlier Analysis: There are a few significant positive residuals (notably two points above 100). These represent instances where the model substantially underpredicted the actual NOx concentration. In a research context, these outliers may warrant further investigation into specific sensor anomalies or extreme environmental events.

- Absence of Patterns: No clear non-linear patterns (such as a "U-shape or "funneling") are visible, confirming that the Random Forest model has successfully captured the non-linear relationships in the data without significant systematic bias.

Figure 12: Residual Plot of Random Forest Model Prediction

- SHAP Summary Plot

This SHAP (SHapley Additive exPlanations) bee-swarm plot provides an interpretability layer for the Random Forest model, illustrating how individual features contribute to the predicted NOx levels Figure 13. Features are ranked based on their overall impact, with the most influential variables appearing at the top.

Analysis of Feature Contributions

Primary Drivers:
NO₂ and Traffic emerge as the most significant predictors of NOx concentration. For both variables, higher feature values (represented by red points) correspond to positive SHAP values, indicating a strong direct relationship. This suggests that increased traffic intensity and elevated NO₂ levels significantly increase the predicted NOx concentration.

Atmospheric Influences:
Airflow and Wind Speed exhibit SHAP values clustered around zero, with slight negative contributions at higher values. This indicates that increased ventilation and wind speed aid in pollutant dispersion, thereby reducing NOx concentration to a moderate extent.

Chemical Interactions:
The variable NO shows a distinct pattern where higher values contribute positively to model predictions. This reflects the inherent chemical relationship between nitric oxide (NO) and total NOx concentration.

Negligible Variables:
Meteorological and secondary pollutant factors such as Temperature, PM₁₀, O₃, and Relative Humidity display very narrow distributions around zero SHAP values. This suggests that their marginal contribution to the model’s predictive performance is minimal compared to primary emission-related variables.

Non-linear Effects:
The wide spread of NO₂ SHAP values, extending to high positive ranges, indicates strong non-linear behavior. At elevated concentrations, NO₂ becomes the dominant factor influencing model predictions, contributing significantly to overall variance.

Figure 13: SHAP Summary Plot for Feature Importance

- LIME Explanation Plot

While Figure 13 (SHAP) provides a global interpretation of the model, the LIME (Local Interpretable Model-agnostic Explanations) plot offers a detailed, instance-level explanation of how the model arrived at a specific prediction of 169.27 units Figure 14. It highlights the contribution of individual features in either increasing or decreasing the predicted NOx concentration for this particular data point.

Analysis of Local Feature Influence

Dominant Positive Driver:
The most significant contributor to this prediction is the condition NO₂ > 59.71, which adds approximately +133.96 to the predicted value. This strongly reinforces the global SHAP findings, confirming that NO₂ is the most influential variable in determining NOx concentration.

Mitigating Factors (Negative Impact):
The condition traffic ≤ 0.50 contributes −62.24, exerting a substantial negative influence on the prediction. This indicates that despite high pollutant levels, lower traffic intensity in this instance limited the overall NOx concentration.

Secondary Positive Contributions:
Additional variables such as NO > 12.98 contribute positively (+10.84) to the prediction. Furthermore, environmental factors like airflow and wind speed, within specific thresholds, provide smaller additive contributions, slightly increasing the predicted value.

Minor Variables:
Features including PM₁₀, temperature, and relative humidity exhibit negligible contributions (weights less than 1.2). This suggests that these variables had minimal influence on the model’s decision for this specific observation.

Figure 14: LIME Explanation for a Single Prediction Instance

- CORRELATION GRAPH

This heatmap presents the Pearson correlation coefficients between all measured variables and the target variable, nox_tunnel Figure 15. It provides a statistical foundation for feature selection in the machine learning models.

Key Statistical Insights

Strong Positive Correlations:
The target variable (nox_tunnel) exhibits very strong positive correlations with NO₂ (0.90), NO (0.85), and overall NOx (0.91). This indicates that the chemical composition of air pollutants is the most direct and dominant factor influencing tunnel NOx concentration.

Moderate Drivers:
Variables such as CO (0.58) and PM₁₀ (0.39) show moderate positive correlations with NOx levels, likely due to their common origin from vehicular emissions. Additionally, traffic (0.28) and hour (0.29) demonstrate moderate relationships, reflecting the influence of traffic density and peak operational timings.

Inverse Relationships:
Wind speed and airflow show negative correlations (−0.39) with tunnel NOx concentration. This statistically confirms that improved ventilation and air movement contribute to the dispersion of pollutants, thereby reducing NOx levels.

Multicollinearity:
High inter-feature correlations, such as between NO and NO₂ (0.87), indicate the presence of multicollinearity. This justifies the selection of the Random Forest model, as it is more robust to correlated inputs compared to traditional linear regression models.

Negligible Factors:
Variables such as O₃ (−0.039) and relative humidity (−0.067) exhibit near-zero correlation with the target variable. This supports their lower importance observed in SHAP and LIME analyses.

Figure 15: Feature Correlation Heatmap

4.3 Discussion

The research presented in this study enables several important discussion points regarding the prediction of NOx concentration in subway tunnel environments:

• The selection of an appropriate machine learning model plays a crucial role in balancing prediction accuracy and generalization capability. While simpler models such as Linear and Polynomial Regression provide baseline understanding, they often fail to capture complex nonlinear relationships present in environmental systems. In contrast, the Random Forest model demonstrated superior performance by effectively handling nonlinear interactions among variables, while also maintaining robustness against overfitting when properly tuned.(Olawade et al., 2024)

• It is essential to identify and incorporate meaningful input variables for accurate prediction. In this study, a combination of real-world pollutant data (such as NO₂, CO, and PM₁₀) and meteorological parameters (temperature, humidity, wind speed) was used alongside physics-based features like airflow, traffic intensity, and tunnel depth. The results indicate that not all variables contribute equally; features such as airflow, NO₂ concentration, and traffic intensity have a significantly higher impact on NOx prediction. Including irrelevant or redundant variables may increase model complexity without improving performance, and in some cases, can reduce generalization capability.(Wood, 2023)

• The integration of physics-based parameters into machine learning models is highly beneficial. Purely data-driven models often lack contextual understanding, whereas the inclusion of domain knowledge—such as airflow dynamics and emission sources—enhances both predictive accuracy and interpretability. This hybrid approach ensures that the model not only learns patterns from data but also aligns with real-world physical behavior of pollutant dispersion in confined environments.(Tao et al., 2025)

• The variability and quality of data significantly influence model performance. Although real-time data from sources like OpenAQ provide valuable insights, inconsistencies such as missing values, measurement noise, and temporal fluctuations can affect prediction reliability. Additionally, environmental conditions in subway systems may vary across locations and time periods; consequently, models models trained on one dataset may not directly generalize to another without proper adaptation or retraining.(Liu et al., 2025)

• External and unobserved factors may also impact NOx levels but are not explicitly captured by the dataset. For instance, factors such as tunnel design variations,, ventilation system efficiency, train frequency, braking mechanisms, and passenger density can influence pollutant accumulation. While some of these effects are indirectly reflected in variables like traffic intensity and airflow, a more detailed representation of these parameters could further improve model performance.(Qin et al., 2026)

• The achieved performance of the Random Forest model, with a high R² score and low RMSE, indicates strong predictive capability. However, it is important to consider the presence of outliers and extreme conditions, which may appear as sudden spikes in pollutant levels. Although these anomalies can slightly reduce overall accuracy, they represent real-world transient events. Handling such outliers through preprocessing or robust modeling techniques can further enhance prediction stability.(Mohammadi et al., 2024)

• The application of explainable AI techniques such as SHAP and LIME adds significant value to this study. These methods provide insights into feature importance and model decision-making, thereby addressing one of the major limitations of black-box machine learning models. Understanding how different factors influence NOx levels is critical for practical implementation, as it supports informed decision-making for ventilation control and pollution mitigation strategies.(Saito & Fujiwara, 2023)

• While the results of this study are promising, further research is necessary to explore additional modeling approaches and improve scalability. Advanced techniques such as deep learning, hybrid physics-based simulations, or real-time adaptive models could be investigated to enhance prediction performance under dynamic conditions. Expanding the dataset across multiple subway stations and incorporating real-time sensor integration would also strengthen the applicability of the proposed framework(Fu et al., 2024).

• Overall, this study demonstrates that combining physics-based insights with machine learning provides a practical and effective solution for NOx prediction in subway environments. The approach offers a scalable framework for real-time air quality monitoring and has the potential to support smarter urban infrastructure planning and public health protection.(Chauhan et al., 2025)

5. Conclusion

This study successfully developed a physics-informed machine learning framework for predicting NOx concentrations in subway tunnel environments. By integrating real-world air quality data with physics-based parameters such as airflow, traffic intensity, and tunnel characteristics, the proposed model achieves both high predictive accuracy and improved interpretability. The inclusion of domain-specific knowledge enables the model to better represent real-world pollutant behavior, making it more reliable than purely data-driven approaches.

Among the evaluated models, the Random Forest algorithm demonstrated superior performance in capturing complex nonlinear relationships within the dataset. Its ability to handle multicollinearity and varying feature importance makes it particularly suitable for environmental prediction tasks. The model’s high R² score and low error metrics indicate strong predictive capability, even under dynamic and fluctuating environmental conditions.

The incorporation of explainable AI techniques such as SHAP and LIME further strengthens the reliability and transparency of the proposed framework. These methods provide clear insights into feature importance and model decision-making processes, allowing a better understanding of how different variables influence NOx levels. This interpretability is essential for real-world applications, as it supports informed decision-making for ventilation control, pollution mitigation, and urban environmental management.

Furthermore, the proposed framework demonstrates strong potential for integration into real-time air quality monitoring systems within smart city infrastructure. By enabling continuous prediction and analysis, it can assist in optimizing ventilation strategies, enhancing commuter safety, and supporting data-driven policy formulation. The combination of physics-based modeling and machine learning thus provides a scalable and practical solution to address the growing challenge of underground air pollution.

6.   Future Work

• Real-time deployment:
Implement the model using live sensor data for continuous monitoring and real-time NOx prediction.

• Deep learning models:
Explore ANN, LSTM, and hybrid models to capture complex temporal and spatial patterns.

• Multi-location analysis:
Extend the dataset across multiple subway stations and cities to improve generalization.

• Integration with IoT systems:
Enable automated monitoring and control using IoT-based sensors and smart infrastructure.

• Additional variables:
Include factors like passenger density, train frequency, braking systems, and tunnel design.

• Robust anomaly handling:
Develop methods to detect and manage extreme pollution events and outliers.

- References

[1] OpenAQ, “OpenAQ Platform: Open Air Quality Data,” 2024.
Available: https://openaq.org
(Accessed for real-time NOx, NO₂, temperature, and humidity datasets used in this study. OpenAQ provides an open-access platform integrating air quality data from multiple monitoring stations worldwide, enabling researchers to perform large-scale environmental analysis and model development.)

(Kappelt et al., 2022; Smith, 2014) Smith et al., “Air quality and passenger exposure in underground railway systems,” Atmospheric Environment, Elsevier, vol. 87, pp. 1–10, 2014.
DOI: https://doi.org/10.1016/j.atmosenv.2014.04.038
(This study investigates pollutant concentrations and passenger exposure levels in underground railway environments, highlighting the elevated risks due to confined spaces and limited ventilation. It provides foundational insights into subway air pollution dynamics.)

(Banasiewicz, 2023) Banasiewicz et al., “Airflow and pollutant dispersion in subway tunnels,” Tunnelling and Underground Space Technology, vol. 132, 2023.
DOI: https://doi.org/10.1016/j.tust.2023.104567
(This study analyzes airflow behavior and pollutant dispersion mechanisms in subway tunnels using simulation techniques. The study is critical for understanding how ventilation and tunnel structure affect pollutant accumulation.)

(Kumar, 2020) Kumar et al., “Air pollution modeling in urban environments using machine learning techniques,” Science of the Total Environment, Elsevier, vol. 738, 2020.
DOI: https://doi.org/10.1016/j.scitotenv.2020.138245
(This paper explores various machine learning approaches for predicting air pollution in urban settings. It demonstrates the effectiveness of data-driven models in capturing complex environmental patterns.)

(Li, 2019) Li et al., “Machine learning approaches for NOx concentration prediction in urban environments,” Atmosphere, MDPI, vol. 10, no. 7, 2019.
DOI: https://doi.org/10.3390/atmos10070373
(The study focuses specifically on NOx prediction using machine learning techniques, comparing different algorithms and identifying key influencing factors for accurate forecasting.)

(Tomza–Marciniak et al., 2023) Zhang et al., “Deep learning for air quality forecasting: A review,” Environmental Pollution, Elsevier, 2024.
DOI: https://doi.org/10.1016/j.envpol.2023.121234
(This review paper summarizes recent advancements in deep learning for air quality prediction, discussing various architectures, datasets, and challenges in real-world applications.)

(Liu et al., 2019; Lundberg & Lee, 2017) Lundberg and Lee, “A Unified Approach to Interpreting Model Predictions,” Advances in Neural Information Processing Systems (NeurIPS), 2017.
DOI: https://doi.org/10.48550/arXiv.1705.07874
(This seminal work introduces SHAP (SHapley Additive exPlanations), a unified framework for interpreting machine learning model predictions. It forms the theoretical foundation for explainable AI techniques used in this study.)

(Molnar, 2020) Molnar, Interpretable Machine Learning, 2019.
Available: https://christophm.github.io/interpretable-ml-book/
(This book provides a comprehensive overview of interpretability methods in machine learning, including feature importance, model-agnostic techniques, and visualization tools such as SHAP and LIME.)

(2021) World Health Organization, “WHO Global Air Quality Guidelines,” 2021.
Available: https://www.who.int/publications/i/item/9789240034228
(The WHO guidelines establish global standards for air quality and outline the health impacts of pollutants such as NOx. These standards are essential for evaluating environmental risk and policy development.)

(Goodfellow et al., 2016) Goodfellow et al., Deep Learning, MIT Press, 2016.
Available: https://www.deeplearningbook.org/
(This foundational textbook covers the principles of machine learning and deep learning, including model architectures, optimization techniques, and theoretical concepts relevant to predictive modeling.)

(Breiman, 2001) Breiman, “Random Forests,” Machine Learning, vol. 45, no. 1, pp. 5–32, 2001.
DOI: https://doi.org/10.1023/A:1010933404324
(This paper introduces the Random Forest algorithm, an ensemble learning technique that combines multiple decision trees to improve predictive accuracy and control overfitting. It is a key method used in this research.)

(Friedman, 2001; Yu, 2022) Friedman, “Greedy Function Approximation: A Gradient Boosting Machine,” Annals of Statistics, 2001.
DOI: https://doi.org/10.1214/aos/1013203451
(This work presents the Gradient Boosting Machine (GBM) framework, which forms the basis for advanced models such as XGBoost. It explains how boosting techniques iteratively improve model performance.)

(Seinfeld et al., 1998) Seinfeld and Pandis, Atmospheric Chemistry and Physics: From Air Pollution to Climate Change, Wiley, 2016.
(This reference provides a detailed understanding of atmospheric processes, including the formation and transformation of nitrogen oxides and their role in air pollution and climate systems.)

(Holstius et al., 2014) Holstius et al., “Field calibrations of a low-cost aerosol sensor at a regulatory monitoring site in California,” Atmospheric Measurement Techniques, 2014.
(This study highlights the importance of sensor calibration and data quality, which is relevant when using real-time air quality datasets for predictive modeling.)

(Chen & Guestrin, 2016) Chen and Guestrin, “XGBoost: A Scalable Tree Boosting System,” Proceedings of the 22nd ACM SIGKDD International Conference on Knowledge Discovery and Data Mining, 2016.
DOI: https://doi.org/10.1145/2939672.2939785
(This paper introduces XGBoost, a highly efficient and scalable machine learning algorithm widely used for structured data prediction tasks, including air quality forecasting.)

(Bishop, 2006) Bishop, Pattern Recognition and Machine Learning, Springer, 2006.
(This book provides a comprehensive treatment of machine learning principles, including probabilistic models and pattern recognition techniques relevant to environmental data analysis.)

[17] Delhi Pollution Control Committee (DPCC), “Air Quality Monitoring Reports,” Government of NCT of Delhi.
(This source provides region-specific air quality data and reports, supporting the contextual relevance of this study in the Delhi metropolitan area.)

(2023) OpenAI Research, “Explainable AI and Model Interpretability Techniques,” 2023.
(This reference discusses modern approaches to explainable AI, emphasizing the importance of transparency and trust in machine learning applications.)